Temperature Pressure Monitoring System Design

2025

1. Introduction

This document describes the complete Software Development Life Cycle (SDLC) for a multi-threaded embedded application that periodically polls parameters (Temperature, Pressure, Configuration Version) and processes them to send if it exceeds the Threshold values within sampling time.

The system contains:

- A polling thread that periodically (In ms) reads sensor values

- A processing thread that analyses polled values at calculated intervals (In ms)

- A global data structure storing the latest polled values (Temperature, Pressure and Configuration Version)

- Notification logic (If Threshold exceeds).

2. Requirement Analysis

The system reads sensor values periodically, Process the values against threshold, and generates notifications as the alert if value remain out of range for a defined sampling duration.

Sensors: Temperature Sensor, Pressure Sensor

Polling Interval: As per sensor requirement

Processing Interval: As per sensor requirement

Sampling Time (Violation persistent Time) : 400ms

- Temperature (Range: -20°C to 100°C)

- Pressure (Range: 0 to 10,000 Pascals)

- Configuration Version (5 characters)

2.1 Functional Requirements

=> Sensor Polling:

- The system shall poll Temperature and Pressure sensors every poll interval in ms.

- The system shall store polled values to the global data structure.

=> Data Processing:

- The system shall process all the available polled values every process interval in ms.

- Compare each parameter against threshold values.

=>Threshold Violation Detection:

- System shall detect polled value above or below the threshold value.

- The system shall not send notification immediately on the first violation.

=>Sampling Time Logic:

- If violation is persistent above or equal to sampling time, send notification (Sampling Time = 400ms(Temp)

- If violation is not persistent up to sampling time clear violation flag.

3. System Design

High-Level Architecture

- Thread 1: Polling Task

- Thread 2: Processing Task

- Shared Memory: Global Structure storing latest values

- Mutex or lock for thread safety

- Notification Module

Processing Thread (Threshold check)

Polling Thread (Reads Sensors)

Global Structure (Updated Values)

Notification Generation

Low-Level Architecture

Polling Configuration for each parameter:

| Field | Value |
| --- | --- |
| Parameter ID | PARAM_TEMP(eg) |
| Parameter Type | Int Or String |
| Poll Interval | 50ms(for temp),200ms(for pressure) |
| Last poll time | In ms |

Polled once        Set as zero

Read Fn Pointer Read Int

Read Fn Pointer Read String

Processing Configuration:

| Field | Value |
| --- | --- |
| Parameter ID | PARAM_TEMP |
| Processing Time | 200ms(for Temp),400ms (for pressure) |
| Minimum Threshold | -10(temp),500(pressure) |
| Maximum Threshold | 70(temp),6000(pressure) |
| Violation Time | In ms(Incremented by Processing Time) |
| LastProcess time | In ms |
| SamplingTime | 400ms(Temp)/800ms(pressure) |

Global Polled Data Structure:

| Field | Purpose |
| --- | --- |
| Parameter Type | Int or String |
| Value(Int and String) | Holds actual data (int or string) |
| Read Status | Read Failed or Not |

1. Initialization (Start)

Start -> Create 2 threads for polling and process

-> Create 2 structures to represent polling and process in common header file(sensor.h)

-> Create one global structure to store result

-> which includes Parameter Type and Variable to store int or string

2. Polling Thread

-> Create a parse table to store polling-parameter-specific configuration data. It will also store the updated last poll time according to the loop polling.

-> Enter the polling thread.

-> Get the current time using GetTimeMs.

-> Create a for loop to access the parse table members (the loop limit will be the size of the parse table).

-> First implement logic for polled once concept for to check Config Version

-> In first iteration polling interval and polled once will be zero then continue and check param type is int or string

-> IF int call read fn for Temp or Pressure

->IF param type is string call read fn for config version

->Next jump into periodic polling for temp and pressure

-> If the difference between the current system time (in milliseconds) and the last poll time is greater than the poll interval (in milliseconds), invoke the read function and store the new sensor value.Here also check param typte is Int or String

-> Call the set function to store the result in the global structure.

-> Update the last poll time to the current system time.

3. Process Loop

-> Create a parse table to store process-parameter-specific configuration data. It will also store the updated violation time according to the loop process.

-> Enter the process loop.

-> Create a for loop to access the process-parse table members (the loop limit will be the size of the process parse table).

-> Call the GetPolledValue function (which returns polled values) and store the result in a data variable.

-> IF the value in that data variable is above or below the threshold value, increment the violation time by the process interval time.

-> Else, call the normal process function and reset the violation time to zero.

-> IF the violation time is greater than the sampling time, call the notification function and reset the violation time to zero.

-> Store the violation happened time into checkout variable.

1. Initialization

| Step | Description |
| --- | --- |
| Start | Starts Execution |
| Thread Creation | 2 Threads : Polling Thread & Process Thread |
| Structure Creation | Create 2 Structures -> For polling & Process |
| Global Structure | Global Structure store Results(Includes param type & Variable for int or string |

2. Polling Thread

This is the logic for periodically read Sensor data & poll Config data only once

| Step | Description |
| --- | --- |
| Polling Thread Entry | Thread execution begins |
| Parse polling Table | Create parse table for polling config |
| Loop start | Start of periodic polling loop |
| Get Time | Get the current system time in ms |
| Parse Table Inside loop | For Loop -> To ge each member in the polling parse table |
| Time check | Is (current time – Last poll time)>Poll Interval ? |
| Check Polled Once? | Enters Polled once logic(config version check) |
| Poll once logic | Check IF Polled once & Poll interval are zero(1st iteration logic) -> Continue |
| Check Param | Check IF parameter type is INT or STRING |
| Read INT fn(T/P) | IF INT call read fn for Temp/Pressure |
| Read String fn(Config Ver) | IF STRING -> call read fn for Config Ver check |
| Set Polled value | Call set fn to store polled value to Global variable |
| Set Polled Once | Set Polled once to 1 to prevent second entry -> Continue |
| IF Read fails | Print Error Log & Error Value |
| Periodic polling | Periodic polling Section |
| Read Fn(T/P) | Invoke read fn for temp/pressure(also check param type is INT OR String) |
| IF Read fails | Update to a Flag and print error log |
| Store result | Call set fn to store polled values to global structure |
| Update last poll time | Set Last poll time = Current Time |
| Loop End | End of Paese Table loop,Go back to next member |
| Loop back | Go back to loop start for next polling cycle |

3.Process Loop

This is the logic for evaluating the polled sensor data against threshold and generating notifications

| Step | Description |
| --- | --- |
| Process Thread | Thread execution starts |
| Parse Table | Create parse table for process config |
| Loop Start | Start of periodic loop |
| Parse table loop for each member | FOR loop for each member in the process table |
| Get polled value | Call Getpolled value fn to get Polled values and store the result in a structure pointer |
| Check Data above/below Threshold Max or Mini | Check value get from polling is below lower threshold or above max threshold. Print Log error |
| Violation time increment | IF YES: Increment violation time by process interval time |
| Normal process | IF NO: Call the normal process fn-> Print Sensor value and reset Violation time to 0.Update the last process time. |
| Sampling time check | IS Violation time>sampling time? |
| Notify | IF YES: Call the notification fn and reset Violation time to 0,Log error |
| Store checkout | Store the violation happened time into the last process time. |
| Loop end | End of Parse Table loop, go back to next member |
| Loop back | Go back to loop start for next processing cycle. |

4.  Implementation

- Main.c -> Create Threads

- Polling.c -> Read sensor value

- Process.c -> Compare with Threshold

- Sensor.c -> Initialize Sensor with hardcoded values

- Sensor.h -> Global structure

5.Testing

- Developer Testing

- Unit Testing

- Integration Testing

6.Maintenence

- Flash firmware to production devices